Модуль 1

- Создание машин

Создаем виртуальные машины:

CLI

HQ-CLI

HQ-SRV

BR-SRV

На HQ-SRV и BR-SRV НЕ включаем автоконфигурацию сети.

2. Настройка CLI

hostname ClI

Сеть:

nano /etc/network/interfaces

auto lo

iface lo inet loopback

auto ens192

iface ens192 inet static

address 172.16.6.1

netmask 255.255.255.240

gateway 172.16.6.14

DNS:

nano /etc/resolv.conf

nameserver 8.8.8.8

Перезагрузка:

reboot

3. Настройка HQ-CLI

hostname HQ-CLI

Сеть:

nano /etc/network/interfaces

auto lo

iface lo inet loopback

auto ens192

iface ens192 inet static

address 192.168.12.1

netmask 255.255.255.240

gateway 192.168.12.14

DNS:

nano /etc/resolv.conf

nameserver 8.8.8.8

reboot

4. Настройка BR-SRV

Обновление системы:

apt update

apt install sudo openssh-server -y

Создаем пользователя:

useradd -m -u 1010 -s /bin/bash sshuser

passwd sshuser

Пароль:

P@ssw0rd

Даем sudo без пароля:

visudo

Добавить в конец:

sshuser ALL=(ALL) NOPASSWD:ALL

SSH баннер

nano /etc/issue.net

Authorized access only

SSH конфигурация

nano /etc/ssh/sshd_config

Изменить:

Port 2024

AllowUsers sshuser

MaxAuthTries 2

Banner /etc/issue.net

Сеть

nano /etc/network/interfaces

auto lo

iface lo inet loopback

auto ens192

iface ens192 inet static

address 192.168.21.1

netmask 255.255.255.224

gateway 192.168.21.30

DNS:

nano /etc/resolv.conf

nameserver 8.8.8.8

Перезагрузка:

reboot

5. Настройка HQ-SRV

apt update

apt install sudo openssh-server -y

Создаем пользователя:

useradd -m -u 1010 -s /bin/bash sshuser

passwd sshuser

Пароль:

P@ssw0rd

sudo:

visudo

sshuser ALL=(ALL) NOPASSWD:ALL

SSH

nano /etc/issue.net

Authorized access only

nano /etc/ssh/sshd_config

Port 2024

AllowUsers sshuser

MaxAuthTries 2

Banner /etc/issue.net

Сеть

nano /etc/network/interfaces

auto lo

iface lo inet loopback

auto ens192

iface ens192 inet static

address 192.168.11.1

netmask 255.255.255.192

gateway 192.168.11.61

DNS:

nano /etc/resolv.conf

nameserver 8.8.8.8

reboot

6. Настройка ISP

enable

conf t

hostname ISP

int e0/0

ip address 10.101.3.1 255.255.255.0

no shutdown

ip nat outside

int e0/1

ip address 172.16.4.14 255.255.255.240

no shutdown

ip nat inside

int e0/2

ip address 172.16.5.14 255.255.255.240

no shutdown

ip nat inside

int e0/3

ip address 172.16.6.14 255.255.255.240

no shutdown

ip nat inside

Маршрут:

ip route 0.0.0.0 0.0.0.0 10.101.3.254

NAT:

access-list 1 permit any

ip nat inside source list 1 interface e0/0 overload

Сохранить:

wr

7. HQ-RTR

enable

conf t

hostname HQ-RTR

int e0/0

ip address 172.16.4.1 255.255.255.240

no shutdown

ip nat outside

int e0/1

no shutdown

VLAN

int e0/1.100

encapsulation dot1Q 100

ip address 192.168.11.61 255.255.255.192

ip nat inside

int e0/1.200

encapsulation dot1Q 200

ip address 192.168.12.14 255.255.255.240

ip nat inside

int e0/1.999

encapsulation dot1Q 999

ip address 192.168.1.6 255.255.255.248

ip nat inside

NAT:

access-list 1 permit any

ip nat inside source list 1 interface e0/0 overload

Маршрут:

ip route 0.0.0.0 0.0.0.0 172.16.4.14

wr

8. BR-RTR

enable

conf t

hostname BR-RTR

int e0/0

ip address 172.16.5.1 255.255.255.240

no shutdown

ip nat outside

int e0/1

ip address 192.168.21.30 255.255.255.224

no shutdown

ip nat inside

access-list 1 permit any

ip nat inside source list 1 interface e0/0 overload

ip route 0.0.0.0 0.0.0.0 172.16.5.14

wr

9. GRE + IPSec

HQ-RTR:

conf t

username net_admin privilege 15 secret P@$$word

interface tunnel0

ip address 192.168.254.1 255.255.255.252

tunnel source e0/0

tunnel destination 172.16.5.1

tunnel mode gre ip

ACL:

ip access-list extended GRE-TRAFFIC

permit gre host 172.16.4.1 host 172.16.5.1

ISAKMP:

crypto isakmp policy 10

encr aes 256

hash sha512

authentication pre-share

group 24

crypto isakmp key cisco123 address 172.16.5.1

IPSEC:

crypto ipsec transform-set IPSEC esp-aes esp-sha-hmac

mode tunnel

crypto map VPN-MAP 10 ipsec-isakmp

set peer 172.16.5.1

set transform-set IPSEC

match address GRE-TRAFFIC

int e0/0

crypto map VPN-MAP

wr

10. OSPF

HQ-RTR

router ospf 1

network 192.168.1.0 0.0.0.7 area 0

network 192.168.11.0 0.0.0.63 area 0

network 192.168.12.0 0.0.0.15 area 0

network 192.168.254.0 0.0.0.3 area 0

passive-interface default

no passive-interface tunnel0

interface tunnel0

ip ospf authentication

ip ospf authentication-key cisco123

BR-RTR

router ospf 1

network 192.168.21.0 0.0.0.31 area 0

network 192.168.254.0 0.0.0.3 area 0

passive-interface default

no passive-interface tunnel0

interface tunnel0

ip ospf authentication

ip ospf authentication-key cisco123

11. Проверка VPN

show crypto isakmp sa

show crypto ipsec sa

12. Проверка SSH

С HQ-CLI

ssh -p 2024 sshuser@192.168.11.1

ssh -p 2024 sshuser@192.168.21.1

13. DHCP

HQ-RTR

conf t

ip dhcp excluded-address 192.168.12.14

ip dhcp pool HQ-CLI-POOL

network 192.168.12.0 255.255.255.240

default-router 192.168.12.14

dns-server 192.168.11.1

domain-name au-team.irpo

14. DNS на HQ-SRV

Установка:

apt install bind9 dnsutils -y

nano /etc/bind/named.conf.options

Добавить:

forwarders {

8.8.8.8;

};

recursion yes;

allow-query { any; };

listen-on { any; };

Перезапуск:

systemctl restart bind9

systemctl status bind9

15. Проверка DNS

ping ya.ru

МОДУЛЬ 2-3

# 1. RAID5 на HQ-SRV

Установка:

apt update
apt install mdadm -y

Проверяем диски

lsblk

Создаем RAID5

mdadm --create /dev/md0 --level=5 --raid-devices=3 /dev/sdb /dev/sdc /dev/sdd

Сохраняем конфиг

mdadm --detail --scan >> /etc/mdadm/mdadm.conf
update-initramfs -u

Создаем файловую систему

mkfs.ext4 /dev/md0

Создаем точку монтирования

mkdir /raid5
mount /dev/md0 /raid5

Автомонтирование

nano /etc/fstab

Добавить:

/dev/md0 /raid5 ext4 defaults 0 0

Проверка

df -h

# 2. NFS сервер

apt install nfs-kernel-server -y

Создаем каталог

mkdir /raid5/nfs
chmod 777 /raid5/nfs
chown nobody:nogroup /raid5/nfs

Экспорт

nano /etc/exports

/raid5/nfs 192.168.12.0/28(rw,sync,no_subtree_check)

Применяем

exportfs -ra
systemctl restart nfs-kernel-server

Проверка

systemctl status nfs-kernel-server

# 3. NTP (Chrony)

Установка

apt install chrony -y

Конфиг

nano /etc/chrony/chrony.conf

Добавить в конец

local stratum 5
allow 0.0.0.0/0
manual

Перезапуск

systemctl restart chrony

Проверка

chronyc clients
date
timedatectl

# 4. Apache + PHP

apt install apache2 -y
apt install php libapache2-mod-php php-mysql php-xml php-gd php-zip php-intl php-curl php-mbstring -y

# 5. MariaDB

apt install mariadb-server -y

Вход

mysql

Создание базы Moodle

CREATE DATABASE moodledb CHARACTER SET utf8mb4 COLLATE utf8mb4_unicode_ci;
CREATE USER 'moodle'@'localhost' IDENTIFIED BY 'P@ssw0rd';
GRANT ALL PRIVILEGES ON moodledb.* TO 'moodle'@'localhost';
FLUSH PRIVILEGES;
exit;

# 6. Установка Moodle

cd /var/www/html
wget https://packaging.moodle.org/stable405/moodle-latest-405.tgz
tar -xzf moodle-latest-405.tgz

Права

chown -R www-data:www-data /var/www
chmod -R 755 /var/www

PHP

nano /etc/php/8.2/apache2/php.ini

Изменить

max_input_vars = 5000

Перезапуск

systemctl restart apache2

# 7. Установка Zabbix

Установка репозитория

wget https://repo.zabbix.com/zabbix/7.0/debian/pool/main/z/zabbix-release/zabbix-release_latest+debian12_all.deb
dpkg -i zabbix-release_latest+debian12_all.deb
apt update

Установка

apt install zabbix-server-mysql zabbix-frontend-php zabbix-apache-conf zabbix-sql-scripts zabbix-agent -y

База

mysql

create database zabbix character set utf8mb4 collate utf8mb4_bin;
create user zabbix@localhost identified by 'P@ssw0rd';
grant all privileges on zabbix.* to zabbix@localhost;
set global log_bin_trust_function_creators = 1;
quit;

Импорт

zcat /usr/share/zabbix-sql-scripts/mysql/server.sql.gz | mysql -uzabbix -p zabbix

Конфиг

nano /etc/zabbix/zabbix_server.conf

DBPassword=P@ssw0rd

Запуск

systemctl enable zabbix-server
systemctl start zabbix-server

# 8. CUPS сервер печати

apt install cups -y

Добавляем администратора

usermod -aG lpadmin $(whoami)

Конфиг

nano /etc/cups/cupsd.conf

Изменить

Listen localhost:631

на

Port 631

Перезапуск

systemctl restart cups

PDF принтер

apt install printer-driver-cups-pdf -y

# 9. NFS клиент (HQ-CLI)

apt install nfs-common -y

Каталог

mkdir /mnt/nfs

fstab

nano /etc/fstab

HQ-SRV.AU-TEAM.IRPO:/raid5/nfs /mnt/nfs nfs defaults 0 0

Монтируем

mount -a
df -h

# 10. Chrony клиент

apt install chrony -y
nano /etc/chrony/chrony.conf

Добавить

server 192.168.11.1 iburst

systemctl restart chrony

# 11. Docker + Wiki (BR-SRV)

Установка

apt install docker.io docker-compose -y
systemctl enable docker
systemctl start docker

Запуск

docker-compose -f /root/wiki.yml up -d

# 12. Ansible

apt install python3-pip sshpass -y
pip install ansible==10.0.0 ansible-pylibssh --break-system-packages

Ключ

ssh-keygen -t rsa

Копирование

ssh-copy-id -p 2024 sshuser@HQ-SRV.AU-TEAM.IRPO
ssh-copy-id root@HQ-CLI.AU-TEAM.IRPO

Hosts

mkdir /etc/ansible
nano /etc/ansible/hosts

[HQ]
HQ-SRV ansible_host=HQ-SRV.AU-TEAM.IRPO ansible_user=sshuser ansible_port=2024
HQ-CLI ansible_host=HQ-CLI.AU-TEAM.IRPO ansible_user=root
HQ-RTR ansible_host=HQ-RTR.AU-TEAM.IRPO ansible_user=net_admin ansible_password=P@$$word ansible_connection=network_cli ansible_network_os=cisco.ios.ios

[BR]
BR-RTR ansible_host=BR-RTR.AU-TEAM.IRPO ansible_user=net_admin ansible_password=P@$$word ansible_connection=network_cli ansible_network_os=cisco.ios.ios

Проверка

ansible all -m ping

# 13. Zabbix agent (BR-SRV)

apt install zabbix-agent -y

Конфиг

nano /etc/zabbix/zabbix_agentd.conf

Server=192.168.11.1

Перезапуск

systemctl restart zabbix-agent

# 14. Проверка SSH (CLI)

ssh sshuser@172.16.4.1 -p 2024
ssh sshuser@172.16.5.1 -p 2024